### YF-B6 & B10  PRODUCT DETAILS

Water flow sensor consists of a copper body, a water rotor, and a hall-effect sensor. When water flows through the rotor, rotor rolls. Its speed changes with different rate of flow. The hall-effect sensor outputs the corresponding pulse signal. This one is suitable to detect flow in water dispenser or coffee machine.

Life is longer than plastic body.

Features

• Compact, Easy to Install

• High Sealing Performance

• High Quality Hall Effect Sensor

• RoHS Compliant

| Type | Dimensions | Working Voltage | Flow Rate Range | Length | Male & Female | Length of Thread | Material |
| --- | --- | --- | --- | --- | --- | --- | --- |
| YF-B6 | DN20 (3/4”) | 5V~15V(DC) | 1~30L/min | 60mm | Double Male | 11mm | Copper |
| YF-B10 | DN25 (1”) | 5V~15V(DC) | 1~30L/min | 60mm | Double Male | 11mm | Copper |

Technical details

| Weight | G.W 152g |
| --- | --- |
| Mini. Wokring Voltage | DC 4.5V |
| Max. Working Current | 15mA (DC 5V) |
| Working Voltage | DC 5V~15V |
| Flow Rate Range | 1~30L/min |
| Frequency | F=(6.6*Q)Q=L/MIN±3% |
| Load Capacity | ≤10mA (DC 5V) |
| Operating Temperature | 0 ~ 80℃ |
| Liquid Temperature | ≤120℃ |
| Operating Humidity | 35%～90%RH |
| Water Pressure | ≤1.75MPa |
| Storage Temperature | -25～+ 80℃ |
| Storage Humidity | 25%～95%RH |
| Error Range | (1~25L\MIN) ±3% |
| Material Description | H57Copper+POM |
| Output Pulse High Level | >DC 4.7V (Input Voltage DC5V) |
| Output Pulse Low Level | <DC 0.5V (Input Voltage DC5V) |
| Output Pulse Duty Cycle | 50%±10% |
| Wire Connection | – Terminal 1 (Red): VDD + – Terminal 2 (Black): GND - – Terminal 3 (Yellow): Signal VOut (GND Referenced/Open Drain , i.e. Requires Pull-Up) |